Prior Art Search Activity

Tasks:

- Perform prior art search using the following parameters, with foam shredder tech as sample:

| Title | GeoVision: Smart Construction Monitoring Using AI and Geotagging |
| --- | --- |
| Short Description | An ESP32-CAM node mounted at a fixed vantage point captures scheduled, geotagged construction-site photographs. A server-side pipeline (OpenCV preprocessing, ResNet18 construction-stage classification, YOLOv8 object-detection corroboration) computes a temporally smoothed, human-gated construction progress percentage, published on a public/owner web dashboard. |
| Date (Search Report Was Made) | August 27, 2026 |
| Keyword Used | construction progress monitoring, AI construction camera, ResNet18 image classification, YOLOv8 object detection, geotagged construction photo, ESP32-CAM construction monitoring, BIM-free progress tracking |

- Use five (5) free websites to conduct your preliminary search report. Paste your specific URL on the third column. See sample below:

| OFFICE | WEBSITE | SPECIFIC URL |
| --- | --- | --- |
| US Patent and Trademark Office | www.uspto.gov | https://ppubs.uspto.gov/pubwebapp/ |
| World Intellectual Property Office | www.wipo.int | https://patentscope.wipo.int/search/en/search.jsf |
| European Patent Office | www.espacenet.com | https://worldwide.espacenet.com/patent/search |
| IP Philippines | www.ipophil.gov.ph | https://onlineservices.ipophil.gov.ph/ |
| Google | www.google.com | https://patents.google.com/ |

- Identify the closest technology (from the search made), list down at least three (3) technologies. Accomplish the table below:

| Keywords Used | Patent Number, Title | Key Claims | Distinction of your Technology over this Patent |
| --- | --- | --- | --- |
| construction progress monitoring, computer vision, deep learning, BIM, construction stage | US 11,972,536 B1 - Monitoring progress of object construction (Dalux ApS; granted Apr. 30, 2024) | A method that (1) provides a digital 3D model of the construction site with object coordinates, (2) captures site photographs with a camera, (3) determines each photograph's location and orientation, (4) renders the 3D model into a per-pixel virtual projection carrying coordinate, surface-orientation, and object-identity data, (5) applies a trained machine-learning model to detect whether each modeled object is present in the photograph and to classify its construction stage, and (6) compares the result against the 3D model to present per-object progress to the user. | GeoVision does not build or require a 3D BIM model of the site at all. It classifies the overall visible construction stage of a single fixed-angle geotagged photo directly with a ResNet18 CNN (10 fine-grained classes folded into 4 macro stages), corroborates the reading with YOLOv8 object detection, and derives one site-wide progress percentage through temporal aggregation (per-camera daily median -> weighted multi-camera mean -> EMA -> monotonic ratchet) capped at 80%, with the final 20% requiring a human inspector's sign-off. It is a low-cost, single-photo CNN pipeline meant for small/medium sites that never had a BIM model, whereas US 11,972,536 depends on an as-planned 3D digital model and per-object image-to-model registration. |
| construction progress, augmented reality, building information model, machine learning, image-based monitoring | US 9,070,216 B2 - Four-dimensional augmented reality models for interactive visualization and automated construction progress monitoring (Univ. of Illinois; granted Jun. 30, 2015) | A method of generating an as-built 3D/4D point-cloud model from unordered daily jobsite photographs, superimposing it in an augmented-reality view on the as-planned 4D BIM/schedule model, and applying a probabilistic (Bayesian/SVM) model to automatically flag and visualize deviations between planned and actual construction progress. | GeoVision does not reconstruct a 3D/4D point cloud or require an as-planned BIM schedule to diff against. It relies on one fixed ESP32-CAM per vantage point capturing scheduled geotagged frames and a supervised CNN classifier trained directly on labeled construction-stage images, so it reports a stage/progress estimate from a single photograph without photogrammetric reconstruction or an existing digital twin - trading fine per-element deviation detection for a simpler, cheaper, near-real-time hardware-plus-CNN pipeline suited to a single-camera undergraduate thesis deployment. |
| construction site, computer vision, equipment detection, productivity, stationary camera | US 11,379,778 B2 - Construction site productivity capture using computer vision (Trimble Inc.; granted Jul. 5, 2022) | A method using stationary image-capturing devices around an earthmoving site to classify heavy equipment (e.g., excavators, dump trucks) in captured images with a trained model, determine each object's pixel location/boundary across a sequence of images, transmit classifications and positions to a remote server, infer the activity being performed, and generate a productivity report - without sensors on the equipment itself. | GeoVision's stationary camera likewise captures scheduled site imagery and applies computer vision, but the target and output differ entirely: GeoVision classifies the construction stage of the built structure itself (site clearing through finishing) to compute an overall percent-complete figure for the owner's dashboard, whereas US 11,379,778 tracks the motion and activity of heavy equipment to estimate operator productivity/utilization. It never classifies construction stage or aggregates a project-wide progress percentage, and needs no HMAC-paired uploader device, GPS geotag, or 4-macro-stage classification scheme. |